Base fixing instructions for the SS0990 Cashier Deposit series

Thank you for purchasing a Phoenix Safe. Your safe is ready prepared for base fixing. To Base fix your safe, follow instructions below.

# HOW TO BOLT YOUR SAFE DOWN –

# YOU WILL REQUIRE

- An electric drill.

- 1masonry drill bit – 1 x 13mm.

- Adhesive tape.

- A pencil

- A ruler

# CHOOSING A PLACE TO INSTALL YOUR SAFE

- Look for a place that does not restrict access and the swing of the door.

- Ensure that the floor is level.

- For best results a concrete floor is preferable.

# DRILLING & BOLTING

- Manoeuvre the safe into the required position. Now with the pencil mark the floor around the safe. (If the safe is against a wall then just the front and sides needs to be marked)

- Open the safe door.

- Mark the positions on the floor for drilling.  Use the pencil through the base fixing holes in the base of the safe.

- Now move the safe to one side, giving you access to the markings on the floor

- Before using the 13mm drill bit. With the ruler measure 95mm from the tip of the bit and put a piece of adhesive tape around the bit to ensure you do not drill too deep.

- Drill the 13mm holes in the floor until you reach your 95mm marker.

- Place the safe back into position and insert the complete bolts through the base of the safe into the drilled holes and tighten.

YOUR SAFE IS NOW BASE FIXED.