Base fixing instructions for the SS1180 Fortress Series

Thank you for purchasing a Phoenix Safe. Your safe is ready prepared for base fixing. To base fix your safe, follow instructions below. If you do not plan to base fix your safe please insert the black base plugs into the fixing holes.

# HOW TO BOLT YOUR SAFE DOWN

# YOU WILL REQUIRE

- An electric drill.

- 1 masonry drill bit – 1 x 13mm.

- Adhesive tape.

- A pencil

- A ruler

- A 6mm Allen key

# CHOOSING A PLACE TO INSTALL YOUR SAFE

- Look for a place that does not restrict access and the swing of the door.

- Ensure that the floor is level.

- For best results a concrete floor is preferable.

# DRILLING & BOLTING

- Manoeuvre the safe into the required position. Now with the pencil mark the floor around the safe. (If the safe is against a wall then just the front and sides needs to be marked)

- Open the safe door.

- Mark the positions on the floor for drilling.  Use the pencil through the base fixing holes in the base of the safe.

- Now move the safe to one side, giving you access to the markings on the floor

- Before using the 13mm drill bit. With the ruler measure 95mm from the tip of the bit and put a piece of adhesive tape around the bit to ensure you do not drill too deep.

- Drill the 13mm holes in the floor until you reach your 95mm marker.

- Clean the floor around the holes.

- There are four parts to your bolts

  - A socket head bolt

  - Two washers (one steel and one plastic)

  - A 40mm spacer

  - 50mm shield

- Take the 50mm shields and the 40mm spacers, insert these into the drilled holes, shield first.

- Place the safe back into position, place the washers onto the bolts and insert the bolts through the base of the safe, into the floor and right down to the shield and tighten until you can not turn the Allen key any further.

- Insert the black base plugs.

YOUR SAFE IS NOW BASE FIXED.